# Lion's Lair Concierge – Product Requirements Document (PRD)

OVERVIEW

Lion's Lair Concierge is a tablet-based, Airbnb-compliant in-unit concierge system.

It delivers a premium guest welcome and guidance experience using local sensors,

a Raspberry Pi hub, and a kiosk web application, with optional cloud extensions.

PRIMARY GOALS

- Detect guest arrival and departure reliably without smart locks or cameras

- Provide a delightful, automated welcome experience

- Operate offline-first with graceful cloud enhancement

- Be extensible toward PMS, AI concierge, and voice features

NON-GOALS (MVP)

- No facial recognition

- No indoor cameras

- No always-listening audio devices

- No PMS or Airbnb API integration

- No AI-generated concierge responses

ARCHITECTURE SUMMARY

Local-first hybrid system:

- Raspberry Pi hub running Home Assistant and a local web server

- Zigbee sensors for door and occupancy detection

- Tablet locked in kiosk mode to a local web application

- Optional cloud service for content sync and future features

SENSOR & PRESENCE MODEL

The system uses a state-based presence model rather than strict directional tracking.

Sensors:

- Door contact sensor on entry door

- Primary occupancy sensor (mmWave preferred) covering living area

- Optional PIR sensor in entry/foyer

Presence States:

- VACANT: No guest detected inside

- OCCUPIED: Guest detected inside

- TRANSITION: Door open or recent movement

State Transitions:

- VACANT → OCCUPIED:

Door opens AND occupancy detected within configurable time window

Trigger: Full welcome experience

- OCCUPIED → OCCUPIED:

Door opens AND occupancy remains

Trigger: Subtle “Welcome back” or no action

- OCCUPIED → VACANT:

Door closes AND no occupancy detected for configurable duration

Trigger: Departure logic (checkout reminders if applicable)

This model prioritizes guest experience reliability over perfect directional accuracy.

MVP FEATURES

1. Arrival Detection (door + occupancy)

2. Tablet Wake and Welcome Screen

3. Manual Guest Name Entry

4. House Manual (rules, how-to, emergency info)

5. Wi-Fi Information

6. Local Area Guide

7. Checkout Instructions

8. Offline-first operation

USER EXPERIENCE (MVP)

- Guest opens door → tablet wakes

- Welcome screen with optional name entry

- Persistent access to concierge menu

- No repeated greetings during short exits/entries

TECHNICAL COMPONENTS (MVP)

- Raspberry Pi 3 or newer

- Home Assistant

- Zigbee USB coordinator

- Local web server hosting concierge app

- Android tablet in kiosk mode

FUTURE ROADMAP

Stage 2:

- QR token-based guest identification

- Cloud-based content management

- Usage analytics

Stage 3:

- PMS integration (Airbnb-compatible via approved providers)

- Text-based AI concierge

Stage 4:

- Voice concierge (guest-initiated, opt-in)

- Optional outdoor doorbell camera (disclosed)

AIRBNB COMPLIANCE PRINCIPLES

- No indoor cameras

- No audio recording or hot-mic behavior

- Clear disclosure of smart devices

- Guest control over interactions

SUCCESS METRICS

- Successful arrival detection rate

- Reduction in host-guest support messages

- Guest engagement with concierge features

- Zero Airbnb policy violations